Impact of Fe content in the reactivity of clays calcined under reductive conditions

Karina Duverger, duvergerkarina@gmail.com, Centro de Investigación y Desarrollo de Estructuras y Materiales (CIDEM)

Fernado Martirena, martirenaf@gmail.com, Centro de Investigación y Desarrollo de Estructuras y Materiales (CIDEM)

Abstract (EN)

The substitution of a percentage of the clinker with Supplementary Cementitious Materials (SCMs) is an alternative used in order to reduce the environmental impact of cement industry. Calcined clays are one of the most promising option among the SCMs due to its widely accessibility and excellent pozzolanic properties shown under specific calcination conditions. However, the color of the cement produced with this type of clays is usually reddish due to the iron (hydr)oxides often present in their composition. This is becoming a matter of concern for the cement producers and a limiting factor for the use of low grade kaolinitic clays as SCM, thus color control techniques need to be implemented. The most common approach to control color is to carry out cooling under reductive conditions to favor the formation of the dark color Fe phase magnetite. For that reason, the present investigation evaluates the properties of three different clays with varying Fe2O3 content, which have been cooled under reductive conditions.

Key Words: calcined clays, color control, iron content, reactivity

Abstract (ES)

La sustitución de un porcentaje del clínquer por Materiales Cementicios Suplementarios (MCSs) es una alternativa utilizada para reducir el impacto ambiental de la industria del cemento. Las arcillas calcinadas son una de las opciones más prometedoras entre los MCSs debido a su amplia accesibilidad y las excelentes propiedades puzolánicas que muestran bajo condiciones específicas de calcinación. Sin embargo, el color del cemento producido con este tipo de arcillas suele ser rojizo debido a los (hidr)óxidos de hierro que suelen estar presentes en su composición. Esto se ha convertido en un motivo de preocupación para los productores de cemento y un factor limitante para el uso de arcillas caoliníticas de baja calidad como MCSs, por lo que se hace necesario implementar técnicas de control de color. El enfoque más común para controlar el color es realizar un enfriamiento en condiciones reductoras para favorecer la formación de magnetita, una fase Fe de color oscuro. Por esa razón, la presente investigación evalúa las propiedades de tres arcillas diferentes con contenido variable de Fe2O3, que se han enfriado en condiciones reductoras.

Key Words: arcillas calcinadas, control de color, contenido de hierro, reactividad