NOMBRE DEL SUB-EVENTO

VII SIMPOSIO INTERNACIONAL DE QUÍMICA

Título

Usos y ventajas del tratamiento magnético en el proceso de fabricación de azúcar

Title

Uses and advantages of the magnetic treatment in the sugar manufacturing process

Guillermo Ribeaux Kindelán1, Fidel Gilart González1

1-Centro Nacional de Electromagnetísmo Aplicado (CNEA), Universidad de Oriente, Cuba, ribeaux@uo.edu.cu, fgg@uo.edu.cu

Resumen:

En este trabajo se evalúa la influencia del Campo Magnético (CM) en el proceso de preparación de los cristales utilizados como semilla en los tachos de una empresa azucarera. El valor medio de la inducción de CM es de 7.6 mT. Se determina la longitud de los cristales, observándose que los que se obtienen bajo la influencia del CM son un 39.18 % más pequeños que aquellos que se obtienen sin la influencia de este. Se calculan parámetros estadísticos que permiten un análisis más completo de las características de la población de los cristales y su distribución granulométrica. Luego, se evalúa la calidad del jugo de caña en el proceso de extracción en cuanto a la presencia de dextrana, fue posible determinar la concentración de la misma en el jugo primario y el mezclado empleando el método turbidimétrico a través de la espectrofotometría. Se realizó un diseño experimental 22 empleando dos niveles de inducción de CM, 102.5 y 230.5 mT, con tiempos de exposición de 60 y 190 min. Los resultados arrojaron que en la empresa seleccionada para el estudio, tanto para el jugo primario como para el mezclado, la concentración de dextrana estaba por encima de lo valores permitidos. Se concluye que esto es debido a la baja calidad de la materia prima por su deterioro y escaso saneamiento de los molinos. Se observó una tendencia a la disminución de la concentración de este polisacárido al exponer el jugo al CM.

Abstract:

In this work, the influence of the Magnetic Field (MF) in the process of preparation of the crystals used as seed in the tachos of a sugar company is evaluated. The average value of the MF induction is 7.6 mT. The length of the crystals is determined, observing that those obtained under the influence of MF are 39.18% smaller than those obtained without the influence of this. Statistical parameters are calculated that allow a more complete analysis of the characteristics of the population of the crystals and their granulometric distribution. Then, the quality of the cane juice in the extraction process is evaluated in the presence of dextran, it was possible to determine the concentration of the juice in the primary juice and the mixing using the turbidimetric method through the spectrophotometry. An experimental design 22 was carried out using two levels of induction of MF, 102.5 and 230.5 mT, with exposure times of 60 and 190 min. The results showed that in the company selected for the study, both for the primary juice and for the mixing, the concentration of dextran was above the allowed values. It is concluded that this is due to the low quality of the raw material due to deterioration and poor sanitation of the mills. A tendency to decrease the concentration of this polysaccharide was observed when the juice was exposed to MF.

Palabras Clave: Tratamiento magnético; Cristalización; Nucleación; Azúcares reductores; Dextrana

Keywords: Magnetic treatment; Crystallization; Nucleation; Reducing sugars; Dextrana